Problem Statement: Uncrewed Ground Systems(UGV) require the capability to autonomously navigate and explore the operational environment.

Lit Review

Domain lit review of use drones in military setting, UGV specific, exploration, scouting

There is a history of using UGVs for explosive ordinance disposal and exploration or hazardous spaces. There have been several exploratory programs through the years but production and deployment numbers remain modest compared to the number of UAVs produced and deployed.

UGVs have been used for logistics by both sides in the Ukrainian conflict.

https://en.wikipedia.org/wiki/THeMIS

It important to remember that space exploration rovers represent the highly realibe, long endurance inverse to the growing pains of the earth-bound systems.

Riya Goja Medium Blog

ROS 2 foxy documentation

ROS 2 allows for robot functionality to be implemented in a modular structure. Nodes can be thought of as actor with associated programed behaviors. Topics allow for nodes to communicate with each other through a publish subscribe relationship. Services are another structure that allows nodes to call for specific server behaviors and responses from other nodes. C++ or python both have ROS libraries that provide an API between python and the ROS framework, the project is constrained to python use only. The content and scope of the code called by node definition allows the use and integration of code integrated into ROS compliant software, instead of with the specific physical implementation of a system in mind.

There are several options of software packages for the navigation and robot movement functionality. Nav2 provides services for robot navigation, location determination, and mapping from sensors. Gazebo and TurtleSim both provide simulation environments for programs to be tested.